MODULE 01 · TEACHER'S GUIDE Audio Recording & Playback Recording from a microphone and playing back digital audio

# Module Overview

In Module 1, students set up the most fundamental building block of the entire project: recording audio from a microphone and playing it back. No AI or internet is needed — this is pure hardware I/O. By the end of the session, students should be able to run a Python script that captures three seconds of audio and plays it back through the speakers.

At a Glance Prerequisites: Python installed, virtual environment created, microphone & speakers connected New files: audio.py, main.py New libraries: sounddevice, numpy Estimated time: 45 – 60 minutes Carries forward: audio.py is reused (nearly unchanged) in every subsequent module

# Learning Objectives

- — Explain what a sample rate is and why 16 kHz is appropriate for speech.

- — Describe the int16 data type and its significance for audio storage.

- — Explain the difference between mono and stereo audio and why mono is used here.

- — Read and trace the audio recording/playback pipeline in Python.

- — Run the program successfully and troubleshoot common device issues.

# Key Libraries Explained

## sounddevice

sounddevice is a Python wrapper around PortAudio, a cross-platform audio I/O library written in C. It lets Python scripts talk directly to the computer's audio hardware — both input (microphone) and output (speakers).

Key functions students will see:

- — sd.InputStream() — opens the microphone as a streaming data source

- — stream.read(frames) — reads a fixed number of audio samples (blocking call)

- — sd.play(audio, samplerate) — sends an array of samples to the speakers

- — sd.wait() — blocks until playback is complete

Teacher note: The concept of a 'blocking' call (one that pauses the program until it finishes) is often new to students. Compare it to input() — the program waits until the user is done before moving on.

## numpy

numpy (Numerical Python) is the foundation of scientific computing in Python. It provides the ndarray — an efficient multi-dimensional array type. In this project, audio data is stored as a 1D numpy array of 16-bit integers, one integer per sample.

Why not a plain Python list? A list of 48,000 integers would be slow to process and use more memory. numpy arrays are stored as a single contiguous block of memory and support vectorised operations — arithmetic on the entire array at once.

# Core Concepts — Deep Dives

## Digital Audio: What is sampling?

Sound in the physical world is a continuous pressure wave. Computers can only store discrete (step-by-step) values, so we 'sample' the wave — measure its amplitude at regular intervals. The number of measurements per second is the sample rate, measured in Hz.

| Sample Rate | Quality & Use Case |
| --- | --- |
| 8,000 Hz | Telephone quality — enough for speech, but sounds 'tinny' |
| 16,000 Hz | Used in this project — captures speech up to 8 kHz (Nyquist theorem) |
| 44,100 Hz | CD quality — captures audio up to 22 kHz, overkill for speech recognition |
| 48,000 Hz | Professional video/audio standard |

The Nyquist-Shannon theorem states that to capture audio up to frequency F, you must sample at 2F or higher. Human speech sits between 80 Hz and 8 kHz, so 16 kHz is the minimum sufficient sample rate — and exactly what Whisper is optimised for.

## int16 — the audio data type

Each audio sample is stored as a signed 16-bit integer, meaning values from -32,768 to +32,767. The zero point represents silence. Positive values are pressure above ambient; negative values are below. This gives 96 dB of dynamic range — enough to capture everything from a whisper to a shout without clipping.

Classroom analogy: Think of the microphone membrane as a ruler. At rest it reads 0. A loud sound pushes it to +30,000. A quiet sound moves it to only ±500. We write down that number 16,000 times per second.

# Common Misconceptions

- — 'Higher sample rate = better for this project' — not true. 16 kHz is optimal for speech; higher rates waste memory and processing time without improving transcription accuracy.

- — 'The audio array is the sound file' — the numpy array is raw PCM data in memory, not a file. A file format (like WAV) adds a header describing the data format so other programs can interpret it.

- — 'sd.play() is instant' — play() starts playback but returns immediately. sd.wait() is what actually blocks until the audio finishes. Without wait(), the program might exit before the sound plays.

# Discussion Questions

- — If you doubled the sample rate to 32,000 Hz but kept the recording duration at 3 seconds, what would happen to the size of the audio array? (Answer: it would double to 96,000 samples.)

- — Why do phone calls sound different from in-person conversations even when the connection is perfect? (Hint: telephone audio is typically compressed to 8 kHz.)

- — When would you want stereo audio instead of mono? What information does the second channel add?

- — What would happen if you multiplied all the values in the audio array by 0.5 before playing it back? What about multiplying by 2.0?

# Differentiation

### Support — students who need more scaffolding

- — Walk through the audio pipeline as a diagram: Microphone → sounddevice → numpy array → sounddevice → Speakers.

- — Have students print(len(audio)) and print(audio[:10]) to make the abstract array concrete.

- — Use the analogy of a flipbook: each frame (sample) is one measurement; flipping fast enough creates the illusion of motion (sound).

### Extension — fast finishers

- — Change DURATION and observe the array length. Derive the formula len = rate × duration.

- — Try recording at 8,000 Hz and 44,100 Hz and compare the playback. What do students notice?

- — Write a function that measures the peak volume of a recording by finding max(abs(audio)).

- — Research: what is the Nyquist-Shannon sampling theorem? How does it apply here?

# Setup & Troubleshooting Checklist

| Symptom | Resolution |
| --- | --- |
| No sound recorded | Run 'arecord -l' (Linux) or check Sound settings. Confirm microphone is the default input device. |
| PortAudio not found | 'pip install sounddevice' installs the Python wrapper but PortAudio itself must be installed at the OS level: 'sudo apt install libportaudio2'. |
| Array shape error | stream.read() returns a 2D array even for mono. The .flatten() call in audio.py handles this — ensure students haven't removed it. |
| Playback too quiet/loud | On the Pi, use 'alsamixer' in the terminal to adjust speaker/headphone volume. |
| Module not found | Ensure the virtual environment is activated before running pip install and python main.py. |

# Assessment Answer Key

| Section A — Multiple Choice Answer Key |  |
| --- | --- |
| Q1 | B 16,000 Hz captures speech frequencies up to 8 kHz at minimum cost. Options A, C, D are either too low, too high, or wrong on the reason. |
| Q2 | C int16 stores values from −32,768 to +32,767. float32 (A) and uint8 (B) are wrong types; int64 (D) is unnecessarily large. |
| Q3 | C Mono uses half the data of stereo and is what speech recognition models expect. The Pi can record stereo (A is false); WAV supports stereo (B is false). |
| Q4 | B sd.wait() blocks until audio playback is fully finished. It does not sleep for a fixed time (A), wait for user input (C), or stop recording (D). |
| Q5 | C 48,000 = 16,000 samples/sec × 3 seconds. Common error: students may confuse sample rate with total samples. |
| Section B — Short Answer Marking Guidelines |  |
| Q6 | ✓ Step 1: sd.InputStream opens the microphone at 16,000 Hz, mono, int16. ✓ Step 2: stream.read(FRAMES) blocks until 48,000 samples have been captured. ✓ Step 3: The result is reshaped/flattened into a 1D numpy array. ✓ Step 4 (full marks): The array is returned to main.py for playback or further processing. |
| Q7 | ✓ Halving the sample rate to 8,000 Hz means only frequencies up to 4 kHz are captured. ✓ Speech will still be intelligible but will sound telephone-quality (tinny, less clarity on consonants). ✓ The array will be half the size (24,000 samples for 3 seconds) — trade-off is smaller memory footprint. ✓ Full marks: student identifies both the quality degradation AND the size/performance benefit. |